Cisco Basic LAN – Router & Switch Configuration

Project Overview

A Cisco Packet Tracer basic LAN configuration project based on the supplied topology. The network uses the 172.16.0.0/16 address space and includes one router, two switches, and four PCs.

# Topology

R1 connects to SW1. SW1 connects to PC1, PC2, and SW2. SW2 connects to PC3 and PC4.

| Device | Interface | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- | --- |
| R1 | G0/0 | 172.16.0.254 | 255.255.0.0 | — |
| PC1 | Fa0 | 172.16.0.1 | 255.255.0.0 | 172.16.0.254 |
| PC2 | Fa0 | 172.16.0.2 | 255.255.0.0 | 172.16.0.254 |
| PC3 | Fa0 | 172.16.0.3 | 255.255.0.0 | 172.16.0.254 |
| PC4 | Fa0 | 172.16.0.4 | 255.255.0.0 | 172.16.0.254 |

# Configuration Tasks

Configure the hostname of R1, SW1, and SW2.

Configure the appropriate IP addresses on R1, PC1, PC2, PC3, and PC4.

Manually configure speed and duplex on interfaces connected to other networking devices.

Configure appropriate descriptions on each interface.

Disable interfaces that are not connected to other devices.

# Files in This Project

R1-config.txt – Cisco IOS configuration for router R1.

SW1-config.txt – Cisco IOS configuration for switch SW1.

SW2-config.txt – Cisco IOS configuration for switch SW2.

PC-IP-addressing.txt – IP addressing information and Packet Tracer instructions for PC1–PC4.

topology.png – Image of the network topology.

README.md – GitHub project documentation.

# Verification

After configuration, test connectivity from each PC using:

ping 172.16.0.254

ping 172.16.0.1

ping 172.16.0.2

ping 172.16.0.3

ping 172.16.0.4

Useful Cisco verification commands:

show ip interface brief

show interfaces status

show running-config

# Project Note

The switch configurations assume a Cisco 2960-style interface layout with FastEthernet0/1–FastEthernet0/24 and GigabitEthernet0/1–GigabitEthernet0/2, matching the supplied topology. The actual Cisco Packet Tracer .pkt file should be saved separately and uploaded to the GitHub repository if available.